Starting on February 19, 2021, the format of the headers of the exported FITS files produced changed. Before this date, the files produced were not FITS-compliant. After this date, the files produced were compliant with version 4.0 of the FITS Standard (https://fits.gsfc.nasa.gov/standard40/fits_standard40aa-le.pdf). In addition to changes needed to achieve compliance, several other changes were made to enhance readability (in preparation for final archiving).

A description of the changes made follows.

FITS Keyword Changes

--------------------

Many of the following changes were made so that the exported FITS files are now

FITS-standard compliant. Some changes were made to enhance readability of the FITS

header, or to retain data-series information that would otherwise be lost.

+ keyword value comments

- previously, keyword values lacked comments; example:

RSUN_OBS= 967.630554

- currently, a comment is included, with the following syntax

(square brackets denote optional components):

a '/' character occupies the byte after the byte occupied by

the last value character

a ' ' character occupies the byte after the byte occupied by

the '/' character

[ a string describing the physical units, surrounded by square brackets,

occupies the bytes after the byte occupied by the last ' ' character;

a ' ' occupies the byte after the byte that the ']' occupies ]

(if the value has a physical unit)

a comment occupies the bytes after the byte that the last ' ' occupies

[ a ' ' occupies the byte after the byte that the last comment character

occupies; a string containing the DRMS keyword name in curly braces

occupies the bytes after the byte that the last ' ' occupies ] (if

the FITS keyword name does not equal the DRMS keyword name - to retain

the DRMS the mapping from FITS keyword name to DRMS keyword name)

- current examples:

RSUN_OBS=   945.91434453710849 / [arcsec] Apparent radius of the Sun seen by SDO

DATE-OBS= '2016-05-09T10:22:43.900' / [ISO] Observation date {DATE__OBS}

+ number keyword values

- previous, number keyword values were not right-justified in bytes 11 through 30

(justification is a FITS requirement); example:

RSUN_OBS= 967.630554

- currently, number values are right-justified; example:

RSUN_OBS=   945.91434453710849 / [arcsec] Apparent radius of the Sun seen by SDO

+ missing keyword values

- previously, NaN keyword values, the value to indicate a missing floating-point value

in DRMS, were represented by a non-quoted value of three characters: nan; this was

not FITS-compliant; example:

OSCNMEAN= nan

- previously, non-floating point keyword values were represented by compliant numbers,

like -2147483648 for a missing integer value; example:

ROI_LLY1=          -2147483648

- currently, a consistent, FITS-compliant method is used to denote all missing values,

regardless of data type:

a FITS keyword name (1 to 8 chars in length only) occupies the first 8 bytes

an '=' character occupies byte 9

a ' ' (a space) character occupies byte 10

' ' characters occupy the next several bytes (determined by FITSIO keyword-

writing routines)

a '/' character occupies the byte after the byte occupied by the last ' '

character

a ' ' character occupies the byte after the byte occupied by the '/' character

the string "(MISSING)" occupies the next 10 bytes after the byte occupied by

the '/' character

a ' ' character occupies the byte after the byte that the last ')' character

occupies

a comment occupies the bytes after the byte that the ' ' occupies

[ a ' ' occupies the byte after the byte that the last comment character

occupies; a string containing the DRMS keyword name in curly braces occupies

the bytes after the byte that the last ' ' occupies ] (if the FITS keyword

name does not equal the DRMS keyword name - to retain the DRMS the mapping

from FITS keyword name to DRMS keyword name)

- current example:

CALVER32=                      / (MISSING) Calibration Version {ROI_NWIN}

+ date keywords

- previously, FITS-reserved date keyword values (DATE, DATE-OBS, etc.) were

not consistent (various representations were used: ISO 8601, TAI, with/without

time zones, depending on data series); the Standard requires that the value of

these keyword be have the form YYYY-MM-DDThh:mm:ss[.sss...] WITHOUT a time zone

specified; the value is interpreted as a UTC time; it should not be interpreted

as an ISO time (which would imply the string was local time)

- currently, dates are all represented by UTC times of the form YYYY-MM-DDThh:mm:ss[.sss...];

examples:

DATE    = '2017-11-03T13:31:59.000' / [ISO] HDU creation date

DATE-OBS= '2016-05-09T10:22:43.900' / [ISO] Observation date {DATE__OBS}

+ keyword exponent-notation values

- previously, floating-point exponents were denoted by lower-case 'e' and 'd'

characters, which are not FITS-compliant

- currently, upper-case 'E' and 'D' are used

+ long keyword values

- previously the length of certain string keywords, like `SOURCE`, exceeded the

number of bytes available so their values were truncated; example:

SOURCE  = 'hmi.lev1[:#195382928,#195382880,#195382832,#195382784,#195382736,#19'

- currently, the FITS long-string convention is used (the string continues to the

next line with one or more CONTINUE keywords; each line, other than the last one, ends in

an '&'); example:

SOURCE  = 'hmi.lev1[:#195382928,#195382880,#195382832,#195382784,#195382736,#1&'

CONTINUE  '95382688,#195382640,#195382592,#195382964,#195383024,#195383072,#19&'

CONTINUE  '5383120,#195383168,#195383216,#195382929,#195382881,#195382833,#195&'

...

CONTINUE  '3199,#195383247]'   / level 1 filtergrams used to produce the observa

+ precision of floating-point keyword values

- previously, a user-defined format string was used to generate the values; example:

DATAMEAN= -1473.85

- currently, the "maximum" precision used the FITSIO writing routines is used;

example:

DATAMEAN=  -1473.8548583984375 / [m/s] Mean value from pixels within 99% of sola

- a separate document will be provided to describe actual precision

+ DRMS ID

- previously, no keyword value uniquely identified each and every data file

- currently, such a keyword, DRMS_ID, uniquely identifies each and every segment

file; its value is generated by concatenating the DRMS series, the series recnum,

and the segment, with colons separating the components; example:

DRMS_ID = 'hmi.V_720s:485458:Dopplergram' / DRMS ID